DOCUMENTATION FILE

COPYRIGHT (C) 1988, 1989

ARRICK ROBOTICS

ALL RIGHTS RESERVED

PROGRAM NAME:	MD-2SUBQ.BAS

DESCRIPTION:	MD-2 QUICK-BASIC SUBROUTINES

DOCUMENTATION:	MD-2SUBQ.DOC

PROGRAMMER:     Roger Arrick

LANGUAGE:       QUICK-BASIC V4.0

VERSION:        1.59

EDIT DATE:      1/17/89

EDIT SESSION:   59

Make sure that the version above corresponds to the version number

of the program source code.

MD-2SUBQ.BAS is a set of subroutines written in QUICK-BASIC V4.0 that

can be called (GOSUB'ed) to control the MD-2 dual motor driver.  These

subroutines can easily be ported to most computers with only minor

modifications.  The minor modifications would include different hardware

port addresses and possibly the addition of line numbers.

SUBROUTINE NAME:  MD2.INIT

FUNCTION---------------------------------------------------------------------

Initializes variables used by MD2.MOVE.	DIMENSION statements can be

pulled out and put at the front of your program as required by

QUICK-BASIC.

USAGE------------------------------------------------------------------------

Call (GOSUB) this subroutine when first starting up your program.

Not normally needed in any other situation.

First you must call MD2.INIT to initialize variables, then call

either MD2.ON or MD2.OFF to turn on motor driver and initialize

motor phases, then use MD2.MOVE.

VARIABLES INPUT--------------------------------------------------------------

None

VARIABLES OUTPUT-------------------------------------------------------------

The following variables are initialized.

ADDR.12% = 3BC HEX, base address of MD-2 for motors 1 & 2

ADDR.34% = 378 HEX, base address of MD-2 for motors 3 & 4

ADDR.56% = 278 HEX, base address of MD-2 for motors 5 & 6

CHECK.KEY$(1-6) = "Y", set key checking to YES

CHECK.LIMIT$(1-6) = "Y", set limit checking to YES

CURRENT.POSITION(1-6) = 0, set position to 0

DIRECTION$(1-6) = "F", set direction to forward

LAST.PATTERN%(1-6) = &HE, set last pattern to E hex

MOVE.ACTION$ = "P", set move action to POSITION SEEK

POWER.DOWN$(1-6) = "Y", set power down when done to YES

SPEED%(1-6) = 0, set fastest speed possible

STEPS.TO.MOVE(1-6) = 0, set # of steps to move

STEP.TYPE$(1-6)	= "H", set step type to HALF

TARGET.POSITION(1-6) = 0, set target position to 0

VARIABLES DISTROYED---------------------------------------------------------

I

VARIABLES USED---------------------------------------------------------------

ADDR.12%

3BC HEX, base hardware address of MD-2 for motors 1 & 2.

ADDR.34%

378 HEX, base hardware address of MD-2 for motors 3 & 4.

ADDR.56%

278 HEX, base hardware address of MD-2 for motors 5 & 6.

CHECK.KEY$(MOTOR%)   where MOTOR% is 1 - 6

Controls keyboard checking during motor movement.  "Y"=yes,

"N"=no. If set to "Y" then any keyboard entry will stop the

motor(s) that are moving. If set to "N" then a keyboard entry

will have no affect on motor movement.

CHECK.LIMIT$(MOTOR%)    where MOTOR% is 1 - 6

Controls limit switch checking during motor movement. "Y"=yes,

"N"=no, If set to "Y" then that motor will stop if it's limit

switch comes on during motion.  "N" inhibits the checking of

limit switches.

CURRENT.POSITION(MOTOR%)   where MOTOR% is 1 - 6

This is the current position of each motor relative to the

reverse limit in number of steps.  This variable is updated

in the MD2.MOVE subroutine after moving except when the

continously "C" move action is selected.  No need to modify

this, only read it.

DIRECTION$(MOTOR%)  where MOTOR% is 1 - 6

This is the desired direction of a motor. "F"= forward and

"R"= reverse. It is needed when using the "S" action which

moves the motors a certain number of steps. It is also needed

when using the "C" continously move action.  It is not needed

when using the "P" action which moves the motors to a certain

position but is updated to the direction required to seek the

selected position.  The direction is set to reverse if the

target position is less than the current position.

LAST.PATTERN%(MOTOR%)    where MOTOR% is 1 - 6

This is the last step pattern sent to a motor. It is needed

by the MD2.MOVE subroutine so the correct starting step

pattern is used.  No need to deal with this.

MOVE.ACTION$

This is the action that the MD2.MOVE subroutine will take.

P = move to a certain position.

S = move a certain number of steps.

C = move continously.

POWER.DOWN$(MOTOR%)

This variable controls power-down-when-done.  When set to

"Y" for YES, the motor's phases will be turned off when the

motor's movement has stopped.  This keeps the motors cool

when not in use. "N" = no and will leave the motor's phases

turned on. Leaving the motor's phases on makes the motor hard

to move when stopped (like a brake).

RETURN.STATUS$

Set when a movement command has stopped.

"O" = OK, movement command completed

"L" = movement command interrupted by limit switch

"K" = movement command interrupted by key press

SPEED%(MOTOR%)

This determines the speed of each motor.  0 is the fastest.

Speed is also a function of the computer being used.

Using a speed parameter that is faster than the maximum

motor speed will cause the motor to loose steps or just jump

around.  Experiment with this.

STEPS.TO.MOVE(MOTOR%)

This determines the number of steps to move a motor when

the "S" action is used.  No negative numbers please.  Not

needed when using "P" or "C" move actions.

STEP.TYPE$(MOTOR%)

This controls the type of step pattern used on a motor.

"H"=half step, "D"=double, "S"=single.  Use Half step

whenever possible.  See the manual for a listing of these

step patterns for more information.

TARGET.POSITION(MOTOR%)

This tells MD2.MOVE where to move a motor when using the

"P" action.  The number of steps to move is calculated

from this and the current position.  Numbers can be positive

or negative.  The position is the number of steps relative

to the reverse limit.  See CURRENT.POSITION variable for

more information.

SUBROUTINE NAME:  MD2.ON

FUNCTION---------------------------------------------------------------------

This subroutine turns ON the motor driver circuits and initializes

the motor phases (windings) ON of the motor(s) specified by the

MOTOR% variable.

USAGE------------------------------------------------------------------------

Call (GOSUB) this subroutine when you are starting your program

and you want the motor phases (windings) initialized ON.

First you must call MD2.INIT to initialize variables, then call

either MD2.ON or MD2.OFF to turn on motor driver and initialize

motor phases, then use MD2.MOVE.

VARIABLES INPUT--------------------------------------------------------------

MOTOR%  = 1,2,3,4,5,6,12,34 or 56.  This specifies the motor(s) and

driver.

VARIABLES OUTPUT-------------------------------------------------------------

None.

VARIABLES DISTROYED---------------------------------------------------------

None.

VARIABLES USED---------------------------------------------------------------

ADDR.12%, ADDR.34%, ADDR.56%, MOTOR%

SUBROUTINE NAME:  MD2.OFF

FUNCTION---------------------------------------------------------------------

This subroutine turns ON the motor driver circuits and initializes

the motor phases (windings) OFF of the motor(s)	specified by the

MOTOR% variable.

USAGE------------------------------------------------------------------------

Call (GOSUB) this subroutine when you are starting your program

and you want the motor phases (windings) initialized OFF.  Also call

when your program is done and you want the motors turned off.

First you must call MD2.INIT to initialize variables, then call

either MD2.ON or MD2.OFF to turn on motor driver and initialize

motor phases, then use MD2.MOVE.

VARIABLES INPUT--------------------------------------------------------------

MOTOR%  = 1,2,3,4,5,6,12,34 or 56.  This specifies the motor(s) and

driver.

VARIABLES OUTPUT-------------------------------------------------------------

None.

VARIABLES DISTROYED---------------------------------------------------------

None.

VARIABLES USED---------------------------------------------------------------

ADDR.12%, ADDR.34%, ADDR.56%, MOTOR%

SUBROUTINE NAME:  MD2.MOVE

FUNCTION---------------------------------------------------------------------

The MD2.MOVE subroutine is used to move the motor(s).  To use it,

simply set up the variables and GOSUB.  When done, the position

values will be adjusted and a return status will be given in the

RETURN.STATUS$ variable.  The bulk of this routine is to get the

variables setup for the move loop which is the part of the code

that actually moves the motor(s).  MD2.MOVE can move one or two

motors at a time.  When moving 2 motors both must be on the same

driver, such as # 1 & 2 or # 3 & 4.  Looking at this code will tell

alot about how the MD-2 driver works.  Much of the code can be

removed if not needed such as limit switch checking, key-press

checking or double/single step patterns.  The delay loop after

each step can be removed or changed if variable step rates are

not needed or if the computer is not fast enough.  The code within

the move loop is critical due to the speed in which the step

patterns must be given.  There is a limitation of 1.4 billion steps

per move except in "C" continous mode which has no limit.

Don't modify this code unless you're sure you know what's going on.

USAGE------------------------------------------------------------------------

To use MD2.MOVE just set up the required variables and GOSUB.

The variables listed below in "VARIABLES INPUT" will explain

which ones do what.  Only the variables relevent to your movement

need to be setup.  For instance, if you want to move motor # 3

you don't have to touch the speed variable for the other motors.

The following example will move motor # 3, 400 steps forward, at

the fastest speed using half step mode.

GOSUB MD2.INIT		'Initialize variables.

MOTOR%=34		'Initialize motors 3 & 4.

GOSUB MD2.OFF

CHECK.KEY$(3) = "Y"	'Set key-press checking ON for emergency stop.

CHECK.LIMIT$(3) = "N"	'Limit switch checking not needed.

DIRECTION$(3) = "F"	'Set direction to Forward.

POWER.DOWN$(3) = "Y"	'Turn power to the motor OFF when done.

SPEED%(3) = 0		'Set speed to fastest.

STEPS.TO.MOVE%(3) = 400 'Move 400 steps.

STEP.TYPE$(3) = "H"	'Half step mode.

MOTOR% = 3		'Move motor 3 only.

MOVE.ACTION$ = "S"	'Move a certain # of steps.

GOSUB MD2.MOVE		'Move.

Now to move motor # 3 400 steps reverse at a slower speed.

DIRECTION$(3) = "R"	'Set direction to reverse.

SPEED%(3) = 20		'Slower speed.

GOSUB MD2.MOVE		'Move.

Notice only the new parameters need to be set.

VARIABLES INPUT--------------------------------------------------------------

CHECK.KEY$(MOTOR%)   where MOTOR% is 1 - 6

Controls keyboard checking during motor movement.  "Y"=yes,

"N"=no. If set to "Y" then any keyboard entry will stop the

motor(s) that are moving. If set to "N" then a keyboard entry

will have no affect on motor movement.

CHECK.LIMIT$(MOTOR%)    where MOTOR% is 1 - 6

Controls limit switch checking during motor movement. "Y"=yes,

"N"=no, If set to "Y" then that motor will stop if it's limit

switch comes on during motion.  "N" inhibits the checking of

limit switches.

CURRENT.POSITION(MOTOR%)   where MOTOR% is 1 - 6

This is the current position of each motor relative to the

reverse limit in number of steps.  This variable is updated

in the MD2.MOVE subroutine after moving except when the

continously "C" move action is selected.  No need to modify

this, only read it.

DIRECTION$(MOTOR%)  where MOTOR% is 1 - 6

This is the desired direction of a motor. "F"= forward and

"R"= reverse. It is needed when using the "S" action which

moves the motors a certain number of steps. It is also needed

when using the "C" continously move action.  It is not needed

when using the "P" action which moves the motors to a certain

position but is updated to the direction required to seek the

selected position.  The direction is set to reverse if the

target position is less than the current position.

MOTOR%

This is the motor # to use when the MD2.MOVE subroutine

is called.  For single motor operation this should be set

to 1,2,3,4,5 or 6.  See ADDR variable for more information.

For double motor operation this should be set to 12, 34 or

56.  If set to 12 then motors 1 and 2 are moved and so-on.

MOVE.ACTION$

This is the action that the MD2.MOVE subroutine will take.

P = move to a certain position.

S = move a certain number of steps.

C = move continously.

When moving 2 motors at the same time (ex: MOTOR%=34) the

same move action is used for both motors.

POWER.DOWN$(MOTOR%)

This variable controls power-down-when-done.  When set to

"Y" for YES, the motor's phases will be turned off when the

motor's movement has stopped.  This keeps the motors cool

when not in use. "N" = no and will leave the motor's phases

turned on. Leaving the motor's phases on makes the motor hard

to move when stopped (like a brake).

SPEED%(MOTOR%)

This determines the speed of each motor.  0 is the fastest.

Speed is also a function of the computer being used.

Using a speed parameter that is faster than the maximum

motor speed will cause the motor to loose steps or just jump

around.  Experiment with this.  When moving 2 motors the

speed parameter of the first motor is used.

STEPS.TO.MOVE(MOTOR%)

This determines the number of steps to move a motor when

the "S" action is used.  No negative numbers please.  Not

needed when using "P" or "C" move actions.

STEP.TYPE$(MOTOR%)

This controls the type of step pattern used on a motor.

"H"=half step, "D"=double, "S"=single.  Use Half step

whenever possible.  See the manual for a listing of these

step patterns for more information.

TARGET.POSITION(MOTOR%)

This tells MD2.MOVE where to move a motor when using the

"P" action.  The number of steps to move is calculated

from this and the current position.  Numbers can be positive

or negative.  The position is the number of steps relative

to the reverse limit.  See CURRENT.POSITION variable for

more information.

VARIABLES OUTPUT-------------------------------------------------------------

CURRENT.POSITION(MOTOR%)   where MOTOR% is 1 - 6

This variable is updated at the end of the movement command

except when the "C" continously MOVE.ACTION$ is used.

Its value is the number of steps relative to the home

position which is the reverse limit.  A minus number

indicates a position which is reverse of the reverse limit.

A positive number indicates a position which is forward of

the reverse limit.  This variable is set to 0 in MD2.INIT.

DIRECTION$(MOTOR%)  where MOTOR% is 1 - 6

This variable is adjusted to the correct direction when

the "P" (move to a certain position) MOVE.ACTION$ is used.

"F" = forward and "R" = reverse.

RETURN.STATUS$

This variable returns the status after a movement command.

The following codes can be found in this variable:

"O" = Movement command completed OK.

"L" = Movement command interrupted by limitswitch

"K" = Movement command interrupted by Key-pressed.

VARIABLES DISTROYED---------------------------------------------------------

The following variables are distroyed by the subroutine so don't

use them.  There is no need to discuss their function.

A%, D%, D1%, D2%, K$, M%, M1%, M2%, L1%, L2%, P%(1-8), PN%(1-8),

I%, S1%, S2%, S3%, S4%, D1%, D2%.

VARIABLES USED---------------------------------------------------------------

All of the variables mentioned above.

END OF DOCUMENTATION