MD-2SUBQ.BAS Quick-Basic Subroutine Library

Version 2.00

Documentation File

Copyright (C) 1988-1990, ARRICK ROBOTICS, All Rights Reserved

The MD-2SUBQ.BAS program file contains a collection of subroutines

which can be 'MERGED' into the user's own program written in Microsoft

Quick-Basic Version 4.0 or later.  These subroutines provide all of the

motion control functions needed for most applications and can be ported

to most other types of BASIC compilers used on MSDOS based computers.

It is not necessary for the user to understand how these subroutines

work, only how to use them.

Subroutine summary --------------------------------------------------------

MD2.INIT   Initializes variables and activates motor(s).  Used at

the beginning of programs.

MD2.MOVE   Move the motor(s).  Used whenever the motor(s) need to

be moved.

MD2.HOME   Home positioning sequence.  Moves the motor(s) to the

home switch then forward until the switch is deactivated.

Used to establish the Home (0) position for reference at

the beginning of the program and whenever recalibration

is needed.  Can only be used when the switch is attached

to the mechanical system at the desired home position.

MD2.SWITCH Read the switch status.  Used when the condition of the

switches need to be known.

MD2.RESET  Reset the ports.  Used at the end of the program to

restore the condition of the parallel printer port(s).

Subroutine library usage --------------------------------------------------

The MD-2SUBQ.BAS Quick-Basic subroutine library is used by first merging

the file onto the end of your motor control program.  You must move the

DIM (dimension) statements from the MD2.INIT subroutine to the beginning

of the program as required by Quick-Basic.

To use the subroutines, set the appropriate parameters and GOSUB.

All parameters used with the subroutines are stored in variables.  The

following list describes these variables.  Make sure when setting a

parameter that capital letters are used when required such as "Y" not "y".

Other variables are used by the subroutines and start with the prefix MD2.

Do not use any variables starting with the letters MD2 to avoid conflicts.

Variables needed by the programmer ----------------------------------------

BACKLASH%(MOTOR%)   where MOTOR% is 1 - 6

Sets backlash compensation.  If the mechanical system

driven by the stepper motor has gear backlash, belt

stretch or some other looseness, then this variable can

be set to compensate for it.  This is done by adding

steps to the step count whenever the direction is changed.

Valid values are integers from 0 to +32,767.  MD2.INIT

sets the default to 0 for no compensation.

CHECK.KEY$(MOTOR%)   where MOTOR% is 1 - 6

Controls keyboard checking during motor movement.  "Y"=yes,

"N"=no. If set to "Y" then pressing the space bar will stop

the motor(s) that are moving. If set to "N" then the space

bar will have no affect on motor movement.  This can be used

for emergency stop purposes.  When moving 2 motors, the

first motor's CHECK.KEY$ parameter is used.  The default

set by MD2.INIT is "Y".

CHECK.SWITCH$(MOTOR%)    where MOTOR% is 1 - 6

Controls switch checking during motor movement. "Y"=yes,

"N"=no, If set to "Y" then that motor will stop if its

switch becomes active during movement.  "N" inhibits the

checking of the	switch.  Normally the switches are used

for Home position reference, but they can be used for

limit switches or general input.  The default is "N".

CURRENT.POSITION&(MOTOR%)   where MOTOR% is 1 - 6

This is the current position of each motor relative to the

Home or 0 position in number of steps.  This variable is

updated in the MD2.MOVE subroutine after moving except when

the continuously "C" move action is selected.  This variable

is set to 0 when using the MD2.HOME subroutine to home the

motor(s).  Range: Integers -2,147,483,648 to +2,147,483,647.

DIRECTION$(MOTOR%)  where MOTOR% is 1 - 6

This is the desired direction of a motor. "F"= forward and

"R"= reverse. It is needed when using the "S" action which

moves the motors a certain number of steps. It is also needed

when using the "C" continuously move action.  It is not needed

when using the "P" action which moves the motors to a certain

position but is updated to the direction required to seek the

selected position.  The direction is set to reverse if the

target position is less than the current position.  "F" is

the default set by MD2.INIT.

MOTOR%

The motor number is use throughout the subroutines to

indicate which motor is to be acted upon.  Up to 3 parallel

printer ports can be connected to an IBM PC style personal

computer allowing 3 MD-2's which can control 6 stepper

motors total.  Motor numbers correspond to the port they

are connected to.  The motors connected to port 3BC hex

are called 1 & 2, port 378 are 3 & 4 and port 278 are 5 & 6.

Valid motor numbers are 1,2,3,4,5,6 or, when used in pairs,

12, 34 or 56.

MOVE.ACTION$

This is the action that the MD2.MOVE subroutine will take.

P = Position move. In this mode, the MD2.MOVE

subroutine accepts information via the TARGET.POSITION&

(MOTOR%) variables, based on this, the correct direction

and step count is calculated from the current position.

Position 0 is considered the Home position, positions

that are positive are forward from Home and positions

that are negative are reverse from Home.  These positions

are in steps.  The current position variable will be

set properly when the movement has finished, even if the

movement was interrupted by a keypress or switch.

"P" is the default set by MD2.INIT.

S = Step count move.  In this mode, the MD2.MOVE subroutine

accepts information from the variables DIRECTION$(MOTOR%)

and STEPS.TO.MOVE&(MOTOR%).  The current position

variable will be set properly when the movement is done

even if the movement was interrupted by a keypress or

switch.

C = Continuous move.  In this mode, the motor(s) move

the direction specified by the DIRECTION$(MOTOR%)

variable until a key is pressed (if key checking is

enabled) or until a switch is pressed (if switch checking

is enabled).  The CURRENT.POSITION&(MOTOR%) variable is

not updated in this mode.

PORT.12$

This variable is set to Y for yes by the MD2.INIT subroutine

if the parallel printer port for motors 1 & 2 at address

3BC hex exists in the computer.  This variable can be used

by the programmer to determine which motor pair to use.

PORT.34$

Same as above but for motors 3 & 4 at address 378 hex.

PORT.56$

Same as above but for motors 5 & 6 at address 278 hex.

POWER.DOWN$(MOTOR%)  where MOTOR% is 1 - 6

This variable controls power-down-when-done.  When set to

"Y" for YES, the motor's phases will be turned off when the

motor's movement has stopped.  This keeps the motors cool

when not in use.  When set to "N" for no will leave the

motor's phases turned on.  Leaving the motor's phases on

makes the motor hard to move when stopped, like a brake.

The default value is "N".

RETURN.STATUS$

This variable is set when the movement has be terminated.

"O" = movement command completed OK

"S" = movement command interrupted by switch

"K" = movement command interrupted by key press

SPEED%(MOTOR%)  where MOTOR% is 1 - 6

This parameter sets the motor's speed.  This is actually the

delay between steps, so when the value is large the motor

will move slow, when small the motor will move fast.

Running the motor too fast will cause lost steps or cause the

motor to jump or vibrate.  As the torque requirements

increase you will need to move slower since stepper motors

have more torque at lower speeds and less torque at higher

speeds.  The speed value is linked to the computer speed and

may act differently on different computers.  Experimentation

will determine the best speed for your application.  When

two motors move together their speeds must be the same.  The

speed of the slowest motor is used if values differ.  Valid

values are integers from 0 to +32,767.  The default is 1000.

STEPS.TO.MOVE&(MOTOR%)  where MOTOR% is 1 - 6

This variable is used in conjunction with the DIRECTION$(

MOTOR%) variable when the "S" action is used for step count

moves.  Valid values are integers from -2,147,483,648 to

+2,147,483,647.  Not needed when using "P" or "C" move

actions.  The default is 0 set by MD2.INIT.

STEP.TYPE$(MOTOR%)  where MOTOR% is 1 - 6

This variable determines the step pattern which is sent to

move the motor(s).  The options are "H" = half step, "D" =

double full step and "S" = single full step.  Normally

half step mode is used since the steps are .9 degrees

(400 steps per revolution) of motor shaft rotation and

resonance problems are reduced which can cause lost steps.

In half step mode the 4 motor phases (windings) are turned

on and off in the following manner:

Phases

Half Step:     Step 1  2  3  4

-----------------

1            X

2         X  X

3         X

4      X  X

5      X

6   X  X

7   X

8   X        X

An X indicates that a phase is on.  Notice in half step

mode that the pattern alternates between one and two phases

being energized.  This causes half steps and medium torque.

In double full step mode two phases are always on.  This

results in maximum torque and 1.8 degree steps (200 steps

per revolution) but also causes more heat and some resonance

problems at certain speeds.  The pattern for double full step

mode is:

Phases

Double Full Step:  Step 1  2  3  4

------------------

1         X  X

2      X  X

3   X  X

4   X        X

In single full step mode only one phase is on at a time.

This results in 1.8 degree steps, low torque and minimum

heat.  The pattern for single full step mode is:

Phases

Single Full Step:   Step 1  2  3  4

-------------------

1                 X

2              X

3           X

4       X

SWITCH$(MOTOR%)	  where MOTOR% is 1 - 6

Status of switches returned from MD2.SWITCH subroutine which

reads the switches and sets these variables.  Y = yes (on),

N = no (off).

TARGET.POSITION&(MOTOR%)  where MOTOR% is 1 - 6

This tells MD2.MOVE where to move a motor when using the

"P" move action.  The number of steps to move is calculated

from this and the current position.  Numbers can be positive

or negative.  The position is the number of steps relative

to the Home or 0 position.  Valid values are integers from

-2,147,483,648 to +2,147,483,647.  See CURRENT.POSITION&

variable for more information.

Variables used by subroutines but not needed by the programmer ------------

Don't use these variable names in your programs.

MD2.ADDRESS%, MD2.ADDR.12%, MD2.ADDR.34%, MD2.ADDR.56%

MD2.BACKLASH.ADJ.1%, MD2.BACKLASH.ADJ.2%, MD2.BACKLASH$

MD2.CHECK.SWITCH.SAVE1$, MD2.CHECK.SWITCH.SAVE2$

MD2.DIRECTION.SAVE1$, MD2.DIRECTION.SAVE2$

MD2.DONE.1$, MD2.DONE.2$,

MD2.I, MD2.I%

MD2.KEYPRESS$

MD2.LAST.PATTERN%(MOTOR%)

MD2.LAST.DIRECTION$(MOTOR%)

MD2.MOVE.ACTION.SAVE$

MD2.HOME.FIRST.MOTOR%, MD2.HOME.SECOND.MOTOR%

MD2.MOTOR.SAVE%

MD2.MOTOR%, MD2.FIRST.MOTOR%, MD2.SECOND.MOTOR%

MD2.PATTERN%, MD2.PATTERN.1%-8%, MD2.PATTERN%(8)

MD2.NEW.PATTERN%(8)

MD2.POWER.DOWN.1$, MD2.POWER.DOWN.2$

MD2.ROTATE%

MD2.STEPS.TO.MOVE.SAVE1&, MD2.STEPS.TO.MOVE.SAVE2&

MD2.SWITCH.1$, MD2.SWITCH.2$

MD2.STEPS.1&, MD2.STEPS.2&

MD2.COMPLETE.1&, MD2.COMPLETE.2&

MD2.TEMP%

MD2.INIT ------------------------------------------------------------------

Description:

The MD2.INIT subroutine is used at the beginning of your program

to initialize the variables with default values and to activate

the motors by initializing the hardware.  The MD2.INIT subroutine

also finds the available ports in the computer and sets the

variables PORT.12$, 34$ and 56$ accordingly. It should not be

necessary to use MD2.INIT elsewhere in the program.

Usage:

It is necessary to use MD2.INIT before using any other MD-2

subroutine.  Only individual motors or motor pairs can be

initialized with each GOSUB MD2.INIT.  To use, simply specify

which motor(s) are going to be moved in the program and call

the subroutine.

Example of single motor initialization:

MOTOR%=3            'GOING TO USE MOTOR 3 ONLY.

GOSUB MD2.INIT      'INITIALIZE VARIABLES AND HARDWARE.

Example of motor pair initialization:

MOTOR%=34           'GOING TO USE MOTORS 3 & 4.

GOSUB MD2.INIT      'INITIALIZE VARIABLES AND HARDWARE.

Example of multiple motor pair initialization:

MOTOR%=12           'GOING TO USE MOTORS 1 & 2.

GOSUB MD2.INIT      'INITIALIZE VARIABLES AND HARDWARE.

MOTOR%=56           'GOING TO USE MOTORS 5 & 6 ALSO.

GOSUB MD2.INIT      'INITIALIZE VARIABLES AND HARDWARE.

Under some unusual circumstance it may be necessary to initialize

The variables but not the hardware.  One example where this is

helpful is in the MD-2MOVE program since the programmer does not

know which motor(s) will be used.  In this case, MOTOR% is set

to 0 which will keep the port from being initialized but still

initialize the variables.  Accidentally initializing a parallel

port that is connected to a printer will disable that printer until

the MD2.RESET subroutine is used.  Port initialization can be done

later in the program using MD2.ON when the MOTOR% is known.

Example when motor to be used is not initially known:

MOTOR%=0             'INITIALIZE VARIABLES ONLY.

GOSUB MD2.INIT

.

.                    'PROGRAM HERE.

.

MOTOR%=34            'NOW I KNOW WHICH MOTORS TO USE.

GOSUB MD2.ON         'INITIALIZE THE HARDWARE ONLY.

Example, check for available ports and initialize them:

MOTOR%=0                 'INITIALIZE VARIABLES ONLY.

GOSUB MD2.INIT

IF PORT.12$="Y" THEN     'PORT FOR MOTORS 1 & 2 AVAILABLE?

MOTOR%=12

GOSUB MD2.INIT

ENDIF

IF PORT.34$="Y" THEN     'PORT FOR MOTORS 3 & 4 AVAILABLE?

MOTOR%=34

GOSUB MD2.INIT

ENDIF

IF PORT.56$="Y" THEN     'PORT FOR MOTORS 5 & 6 AVAILABLE?

MOTOR%=56

GOSUB MD2.INIT

ENDIF

Variables received:

MOTOR%

Variables returned:

The following variables are initialized to default values:

BACKLASH%(MOTOR%)         = 0, set backlash compensation to 0 steps

CHECK.KEY$(MOTOR%)        = "Y", set key checking to YES

CHECK.SWITCH$(MOTOR%)     = "N", set switch checking to NO

CURRENT.POSITION&(MOTOR%) = 0, set current position to 0

DIRECTION$(MOTOR%)        = "F", set direction to forward

MOVE.ACTION$              = "P", set move action to POSITION SEEK

POWER.DOWN$(MOTOR%)       = "N", set power-down-when-done to NO

SPEED%(MOTOR%)            = 1000, set speed which will work on most

computers.

STEPS.TO.MOVE&(MOTOR%)    = 0, set # of steps to move to 0

STEP.TYPE$(MOTOR%)        = "H", set step type to HALF

TARGET.POSITION&(MOTOR%)  = 0, set target position to 0

MD2.MOVE ------------------------------------------------------------------

Description:

The MD2.MOVE subroutine is used to move the motor(s).  Simply

set the required parameters and GOSUB MD2.MOVE.  Any one of the

6 motors can be moved or any pair of motors such as 1 & 2, 3 & 4,

or 5 & 6.

Usage:

To move the motors the programmer must set the parameters then

call the MD2.MOVE subroutine.  Only the parameters that need to

be changed have to be set since the subroutine does not destroy

them.  A variable called RETURN.STATUS$ is set to let the

programmer know how the subroutine was terminated.

Example:

MOTOR%=3               'SET MOTOR TO #3.

MOVE.ACTION$="S"       'MOVE USING STEP COUNT.

DIRECTION$(3)="F"      'FORWARD.

STEPS.TO.MOVE&(3)=400  '400 STEPS.

SPEED%(3)=500          'SET SPEED.

GOSUB MD2.MOVE         'MOVE THE MOTOR.

PRINT RETURN.STATUS$   'DISPLAY THE STATUS.

DIRECTION$(3)="R"      'NOW MOVE REVERSE 400 STEPS.

GOSUB MD2.MOVE

PRINT RETURN.STATUS$   'DISPLAY THE STATUS.

SPEED%(3)=200          'DO IT AGAIN FASTER.

GOSUB MD2.MOVE

PRINT RETURN.STATUS$   'DISPLAY THE MOVE STATUS.

Notice only the parameters that changed or differed from the

defaults were set.

The user has the option of 3 different movement types, 'Position'

moves, 'step count' moves or 'continuous' moves.  This option is

set using the MOVE.ACTION$ variable.

The 'position' move is the most common type.  Set MOVE.ACTION$="P"

and set the TARGET.POSITION&(MOTOR%) variable to the desired position.

The MD2.MOVE subroutine will set the direction and step count

required to move to the target position based on the current position

which is stored in CURRENT.POSITION&(MOTOR%).  This current position

variable can be used by the programmer to find the position.  All

positions are in steps from Home.  A negative number is a position

reverse from home and a positive number forward from home.  This

home position can be set using the MD2.HOME subroutine.  Maximum

positions are -2,147,483,648 and +2,147,483,647, the maximum number

of steps per movement is 2,147,483,647.

Example of a position move:

MOVE.ACTION$="P"            'SET POSITION MOVE.

MOTOR%=3                    'SET MOTOR #.

TARGET.POSITION&(3)=1000    'SET TARGET POSITION.

GOSUB MD2.MOVE              'MOVE.

The 'step count' option is set when MOVE.ACTION$="S". In this mode

the programmer sets the direction in the DIRECTION$(MOTOR%)

variable and the step count in the STEPS.TO.MOVE&(MOTOR%).  The

maximum number of steps that can be moved is 2,147,483,647

Example of a step count move:

MOVE.ACTION$="S"        'SET STEP COUNT MOVE.

MOTOR%=3                'SET MOTOR #.

DIRECTION$(3)="F"       'SET DIRECTION.

STEPS.TO.MOVE&(3)=50    'SET STEP COUNT.

GOSUB MD2.MOVE          'MOVE.

The 'continuous' mode is set when MOVE.ACTION$="C".  In this mode

the motor will move forever or until a key is pressed or a switch

is activated if they are enabled.  The CURRENT.POSITION&(MOTOR%)

variable is not updated during continuous moves.

Example of a continuous move:

MOVE.ACTION$="C"      'SET CONTINUOUS MOVE.

MOTOR%=3              'SET MOTOR #.

DIRECTION$(3)="R"     'SET DIRECTION.

GOSUB MD2.MOVE        'MOVE.

When moving 2 motors the speed of the motors must be the same,

otherwise the speed of the slowest motor is used.  This subroutine

does not accelerate and decelerate the motor(s), this is a limitation

of the current software and not the MD-2 hardware.

Variables received:

BACKLASH%(MOTOR%)

CHECK.KEY$(MOTOR%)

CHECK.SWITCH$(MOTOR%)

CURRENT.POSITION&(MOTOR%)

DIRECTION$(MOTOR%)

MOTOR%

MOVE.ACTION$

POWER.DOWN$(MOTOR%)

SPEED%(MOTOR%)

STEPS.TO.MOVE&(MOTOR%)

STEP.TYPE$(MOTOR%)

TARGET.POSITION&(MOTOR%)

Variables returned:

CURRENT.POSITION&(MOTOR%)  -  except during continuous moves.

RETURN.STATUS$

MD2.HOME ------------------------------------------------------------------

Description:

The MD2.HOME subroutine is used to move the motor(s) to the home

position using the switch.  The motor(s) move reverse until the

switch is activated and then forward until deactivated.  This

sequence causes preloading of belts and bearings in the forward

direction and allows the next motor move to begin without the

switch activated.  The CURRENT.POSITION&(MOTOR%) variable is set

to 0.  This is the home position from which all other movements

will be referenced.  The RETURN.STATUS$ variable is set when

finished to let the programmer know if the movement was successful.

Any single motor or motor pairs can be home'd.  If the RETURN.

STATUS$ variable does not equal 'O' for OK when done indicating a

successful completion, then the CURRENT.POSITION&(MOTOR%) variable

may not be accurate.

Usage:

To use the MD2.HOME subroutine simply set the parameters such

as speed and motor number(s) (if they differ from previous usage)

then GOSUB MD2.HOME.  The home routine is usually used at the

beginning of a program after the initialization to set the 0

reference point for the motors and also during the program when

the motors need to insure their positioning integrity.

Example:

MOTOR%=4            'SET MOTOR #.

GOSUB MD2.HOME      'HOME.

Variables received:

CHECK.KEY$(MOTOR%)

MOTOR%

POWER.DOWN$(MOTOR%)

SPEED%(MOTOR%)

STEP.TYPE$(MOTOR%)

Variables returned:

CURRENT.POSITION&(MOTOR%)

RETURN.STATUS$

MD2.SWITCH -----------------------------------------------------------------

Description:

The MD2.SWITCH subroutine allows the programmer to read the status

of the switches via the variables SWITCH$(MOTOR%).   These switches

are normally used for home reference positioning but can also be used

for limit switches or general purpose inputs.

Usage:

Simply call the subroutine with a GOSUB statement then read the

switch status.

Example:

GOSUB MD2.SWITCH           'READ THE SWITCH STATUS.

IF SWITCH$(1) = "Y" THEN   'SWITCH OF MOTOR # 1 ON?

PRINT "# 1 ON"        'DISPLAY THE RESULTS.

ELSE

PRINT "# 1 OFF"

ENDIF

Variables received:

None

Variables returned:

SWITCH$(1-6)

MD2.RESET -----------------------------------------------------------------

Description:

The MD2.RESET subroutine allows the programmer to return the

parallel printer ports to their previous status.  This turns off

any MD-2's connected to them and enables the use of a printer on

the ports.

Usage:

To use, set the motor number(s) and GOSUB.  To reset all ports, do

GOSUBs using all motors.

Example:

MOTOR%=12             'SET MOTOR # 12, (3BC HEX)

GOSUB MD2.RESET       'RESET THE PORT.

Example to reset all ports:

FOR MOTOR%=1 TO 6     'LOOP THROUGH ALL 6 MOTORS.

GOSUB MD2.RESET  'RESET THE PORT.

NEXT MOTOR%

Variables received:

MOTOR%

Variables returned:

None

'Example Program # 1 --------------------------------------------------------

'The following program demonstrates the use of the Quick-Basic subroutines

'for the MD-2.  This program allows the user to move any motor to any

'position at any speed via keyboard commands.  The source code can be

'found on the MD-2 diskette under the name MD-2EX1.BAS.

'MD-2EX1.BAS  -  EXAMPLE PROGRAM # 1.

'MOVES ANY MOTOR TO ANY POSITION AT ANY SPEED.

START:

'INSERT DIM STATEMENTS FROM MD-2SUBQ.BAS HERE.

'INITIALIZE ALL PORTS

FOR MOTOR% = 1 TO 6

GOSUB MD2.INIT

NEXT MOTOR%

AGAIN:

'DISPLAY SCREEN

CLS

PRINT "MD-2EX1.BAS  -  Example Program # 1"

PRINT "Move any motor to any position at any speed."

'GET MOTOR # AND END IF 0.

INPUT "Enter motor # (1-6) or 0 to end  ", MOTOR%

IF MOTOR% = 0 THEN

'RESET ALL PORTS

FOR MOTOR% = 1 TO 6

GOSUB MD2.RESET

NEXT MOTOR%

END

END IF

'GET SPEED.

INPUT "Enter the speed (0-32767)  ", SPEED%(MOTOR%)

'GET DESIRED POSITION.

INPUT "Enter the desired position (0=HOME)  ", TARGET.POSITION&(MOTOR%)

'HOME THE MOTOR OR MOVE TO A POSITION?

IF TARGET.POSITION&(MOTOR%) = 0 THEN

'HOME

GOSUB MD2.HOME

ELSE

'MOVE

GOSUB MD2.MOVE

END IF

'DISPLAY RESULTS

PRINT "Current position is"; CURRENT.POSITION&(MOTOR%)

PRINT "Return status is "; RETURN.STATUS$

INPUT "Press return to continue ", X$

'DO AGAIN

GOTO AGAIN

'MERGE MD-2SUBQ.BAS SUBROUTINES HERE.

'Example Program # 2 --------------------------------------------------------

'The following example program can be found in the file MD-2EX2.BAS and

'demonstrates a step-and-repeat sequence on an XY positioning table.  First

'the X axis will move to the correct column then the Y axis will move to

'several rows on that column.  This sequence is repeated as many times as

'needed to accomplish the task.  Many automation projects require this type

'of positioning sequence.

'MD-2EX2.BAS - EXAMPLE PROGRAM # 2.

'STEP AND REPEAT SEQUENCE.

START:

'INSERT DIM STATEMENTS FROM MD-2SUBQ.BAS HERE.

'INITIALIZE MOTORS 3 & 4 SINCE THE MD-2 IS CONNECTED TO PORT 378.

MOTOR% = 34

GOSUB MD2.INIT

'SET SOME PARAMETERS DIFFERENT THAN DEFAULTS.

'MOTOR 3 (X AXIS) = SLOW, MOTOR 4 (Y AXIS) = FAST.

SPEED%(3) = 900 : SPEED%(4) = 300

'DISPLAY MESSAGE

CLS : PRINT "STEP AND REPEAT SEQUENCE"

'HOME BOTH MOTORS.

GOSUB MD2.HOME

'QUIT IF HOME ABORTED WITH KEYPRESS.

IF RETURN.STATUS$<>"O" THEN GOTO DONE

FOR X& = 100 TO 1000 STEP 100

'MOVE TO THE RIGHT COLUMN.

MOTOR% = 3

TARGET.POSITION&(3) = X&

GOSUB MD2.MOVE

'QUIT IF MOVE ABORTED WITH KEYPRESS.

IF RETURN.STATUS$<>"O" THEN GOTO DONE

FOR Y& = 10 TO 100 STEP 10

'MOVE TO EACH ROW IN THE CURRENT COLUMN.

MOTOR% = 4

TARGET.POSITION&(4) = Y&

GOSUB MD2.MOVE

IF RETURN.STATUS$<>"O" THEN GOTO DONE

NEXT Y&

NEXT X&

DONE:

PRINT "DONE."

'RESET THE PORTS.

MOTOR%=34

GOSUB MD2.RESET

END

'MERGE MD-2SUBQ.BAS SUBROUTINES HERE.

'Example Program # 3 -------------------------------------------------------

'The following program displays the status of the switches and tells the

'user which ports are available in the computer.  The source code can be

'found in the file MD-2EX3.BAS.

'MD-2EX3.BAS - EXAMPLE PROGRAM # 3.

'DISPLAY STATUS OF SWITCHES AND AVAILABLE PORTS.

START:

'INSERT DIM STATEMENTS FROM MD-2SUBQ.BAS HERE.

'INITIALIZE VARIABLES BUT NOT PORTS (HARDWARE).

MOTOR%=0

GOSUB MD2.INIT

'DISPLAY AVAILABLE PORTS.

CLS

PRINT "----------- AVAILABLE PORTS ----------"

IF PORT.12$="Y" THEN PRINT "PORT 3BC FOR MOTORS 1 & 2 IS AVAILABLE"

IF PORT.34$="Y" THEN PRINT "PORT 378 FOR MOTORS 3 & 4 IS AVAILABLE"

IF PORT.56$="Y" THEN PRINT "PORT 278 FOR MOTORS 5 & 6 IS AVAILABLE"

IF PORT.12$="N" AND PORT.34$="N" AND PORT.56$="N" THEN

PRINT "NO AVAILABLE PORTS"

ENDIF

'DISPLAY SWITCH STATUS UNTIL KEYPRESS.

LOCATE 6,1

PRINT "------ SWITCH STATUS ------"

PRINT " 1    2    3    4    5    6"

LOCATE 12,1

PRINT "PRESS SWITCHES TO TEST, PRESS ANY KEY TO QUIT"

DISPLAY.LOOP:

'READ THE SWITCH STATUS.

GOSUB MD2.SWITCH

'DISPLAY THE RESULTS.

IF PORT.12$="Y" THEN LOCATE 8,2 : PRINT SWITCH$(1);"    ";SWITCH$(2);

IF PORT.34$="Y" THEN LOCATE 8,12: PRINT SWITCH$(3);"    ";SWITCH$(4);

IF PORT.56$="Y" THEN LOCATE 8,22: PRINT SWITCH$(5);"    ";SWITCH$(6);

'QUIT IF KEYPRESS.

IF INKEY$<>"" THEN END

GOTO DISPLAY.LOOP

'MERGE MD-2SUBQ.BAS SUBROUTINES HERE.

'Example Program # 4 -------------------------------------------------------

'The following example executes a preprogrammed position sequence file.  The

'user simply creates a text file of positions and the program reads them and

'moves the motors accordingly.  The position file name used is MD-2EX4.POS.

'The source code can be found in the file MD-2EX4.BAS.

'MD-2EX4.BAS - EXAMPLE PROGRAM # 4.

'READ A TEXT FILE CONTAINING POSITIONS AND MOVE THE MOTORS.

START:

'INSERT DIM STATEMENTS FROM MD-2SUBQ.BAS HERE.

'INITIALIZE MOTORS 1 & 2 SINCE THE MD-2 IS CONNECTED TO PORT 3BC.

MOTOR%=12

GOSUB MD2.INIT

'CHANGE SOME PARAMETERS DIFFERENT THAN DEFAULTS.

SPEED%(1) = 500 : SPEED%(2) = 500

'READ THE FILE UNTIL AN ERROR OCCURS.

CLS

PRINT "READING FILE AND MOVING"

ON ERROR GOTO DONE

OPEN "MD-2EX4.POS" FOR INPUT AS # 1

READ.LOOP:

INPUT #1, TARGET.POSITION&(1), TARGET.POSITION&(2)

PRINT TARGET.POSITION&(1), TARGET.POSITION&(2)

GOSUB MD2.MOVE

'QUIT IF ABORTED BY KEYPRESS.

IF RETURN.STATUS$ = "O" THEN GOTO READ.LOOP ELSE GOTO DONE

DONE:

'RESET THE PORTS AND CLOSE THE FILE.

PRINT "DONE."

CLOSE

GOSUB MD2.RESET

END

'MERGE MD-2SUBQ.BAS SUBROUTINES HERE.

'Example Program # 5 --------------------------------------------------------

'This example program shows the use of 'step-count' moves and allows the

'operator to move motors using the arrow keys.  Source code can be found

'in the file MD-2EX5.BAS.

'MD-2EX5.BAS - EXAMPLE PROGRAM # 5.

'STEP-COUNT MOVE EXAMPLE USING ARROW KEYS.

START:

'INSERT DIM STATEMENTS FROM MD-2SUBQ.BAS HERE.

'INITIALIZE MOTORS 3 & 4 SINCE THE MD-2 IS CONNECTED TO PORT 378.

MOTOR% = 34

GOSUB MD2.INIT

'SET PARAMETERS DIFFERENT THAN DEFAULTS.

SPEED%(3) = 600 : SPEED%(4) = 800

STEPS.TO.MOVE&(3) = 100 : STEPS.TO.MOVE&(4) = 100

MOVE.ACTION$ = "S"    'STEP COUNT MOVES.

CLS : PRINT "USE ARROW KEYS TO CONTROL MOTORS, SPACE BAR TO END"

MOVE.LOOP:

'GET KEYBOARD ENTRY

ENTRY$ = INKEY$

'SPACE BAR = RESET PORT THEN END

IF ENTRY$ = " " THEN

MOTOR% = 34

GOSUB MD2.RESET

END

ENDIF

'UP ARROW = MOTOR 3 FORWARD.

IF ENTRY$ = CHR$(0) + CHR$(72) THEN

MOTOR% = 3

DIRECTION$(3) = "F"

GOSUB MD2.MOVE

END IF

'DOWN ARROW = MOTOR 4 REVERSE.

IF ENTRY$ = CHR$(0) + CHR$(80) THEN

MOTOR% = 3

DIRECTION$(3) = "R"

GOSUB MD2.MOVE

ENDIF

'RIGHT ARROW = MOTOR 4 FORWARD.

IF ENTRY$ = CHR$(0) + CHR$(77) THEN

MOTOR% = 4

DIRECTION$(4) = "F"

GOSUB MD2.MOVE

ENDIF

'LEFT ARROW = MOTOR 4 REVERSE.

IF ENTRY$ = CHR$(0) + CHR$(75) THEN

MOTOR% = 4

DIRECTION$(4) = "R"

GOSUB MD2.MOVE

ENDIF

GOTO MOVE.LOOP

'MERGE MD-2SUBQ.BAS SUBROUTINES HERE.

'Example Program # 6 ------------------------------------------------------

'This example shows the use of continuous movement when setting the

'MOVE.ACTION$ parameter = "C".  This could be used when the motor must

'run forever.  The source code for this program can be found in the file

'MD-2EX6.BAS

'MD-2EX6.BAS - EXAMPLE PROGRAM # 6

'CONTINUOUS MOVEMENT EXAMPLE.

START:

'INSERT DIM STATEMENTS FROM MD-2SUBQ.BAS HERE.

'INITIALIZE MOTOR 3 SINCE THE MD-2 IS CONNECTED TO PORT 378

MOTOR% = 3

GOSUB MD2.INIT

'DISPLAY MESSAGE.

CLS

PRINT "MOVE MOTOR 3 CONTINUOUSLY"

'SET PARAMETERS DIFFERENT THAN DEFAULTS.

MOVE.ACTION$ = "C"     'CONTINUOUS MOVES.

INPUT "ENTER DIRECTION (R=REVERSE, F=FORWARD) ",DIRECTION$(3)

INPUT "ENTER THE SPEED VALUE (0-32767) ",SPEED%(3)

'NOW MOVE FOREVER, QUIT WHEN SPACE BAR PRESSED.

PRINT "MOVING . . . . "

GOSUB MD2.MOVE

'WILL ONLY RETURN IF KEYPRESS.

BEEP

PRINT "MOTION INTERRUPTED BY KEYPRESS"

GOSUB MD2.RESET

END

'MERGE MD-2SUBQ.BAS SUBROUTINES HERE.

End of Documentation